# Network Documentation and Security Evaluation of ACME Inc.

Adam Henderson

Introduction	3

Network Mapping	4

Subnet Table	7

Network Mapping	9

Security Weaknesses	30

Exploitation	30

Mitigation	42

Network Design Critical Evaluation	45

Conclusions	47

Appendices	48

Appendix A – Dirb scan	49

Appendix B – Nmap scan	50

Appendix C – Subnet Calculations	50

Appendix D – Firewall configuration	50

# Introduction

ACME Inc. have recently parted ways with their network manager, on attempt to review documentation for their network they were unable to find any documentation produced about the network. To rectify this critical error ACME Inc. have requested a full network evaluation and security testing. ACME Inc. Have provided the tester with a machine on the network and expect this report to produce the following deliverables.

- A detailed network diagram showing all the devices that are active, in use, and on the network.

- A subnet table which details the subnets used in the network including the subnet address, the subnet mask, the valid range of host addresses for the subnet, the broadcast address of the subnet and the calculations to be provided in the report’s appendix.

- An evaluation of any security weaknesses found including how they were exploited, and what remediations can be put in place to mitigate them. This will also include screenshots to prove the vulnerabilities reproducibility.

- A critical evaluation of the networks design including what’s good, what’s bad and improvements

# Network Mapping

An extensive network map has been provided below, the full page has been used due to its size.

# Subnet Table

| Subnet Address | Subnet Mask | Valid IP Addresses | Broadcast Address |
| --- | --- | --- | --- |
| 13.13.13.0 | 255.255.255.0 or /24 | 13.13.13.1 - 13.13.13.254 | 13.13.13.255 |
| 172.16.221.0 | 255.255.255.0 or /24 | 172.16.221.1-172.16.221.254 | 172.16.221.255 |
| 192.168.0.32 | 255.255.255.224 or /27 | 192.168.0.33 - 192.168.0.62 | 192.168.0.63 |
| 192.168.0.64 | 255.255.255.224 or /27 | 192.168.0.65 - 192.168.0.94 | 192.168.0.95 |
| 192.168.0.96 | 255.255.255.224 or /27 | 192.168.0.97 - 192.168.0.126 | 192.168.0.127 |
| 192.168.0.128 | 255.255.255.224 or /27 | 192.168.0.129 - 192.168.0.158 | 192.168.0.159 |
| 192.168.0.192 | 255.255.255.224 or /27 | 192.168.0.193 - 192.168.0.222 | 192.168.0.223 |
| 192.168.0.224 | 255.255.255.252 or /30 | 192.168.0.225 - 192.168.0.226 | 192.168.0.227 |
| 192.168.0.228 | 255.255.255.252 or /30 | 192.168.0.229 - 192.168.0.230 | 192.168.0.231 |
| 192.168.0.232 | 255.255.255.252 or /30 | 192.168.0.233 - 192.168.0.234 | 192.168.0.235 |
| 192.168.0.240 | 255.255.255.252 or /30 | 192.168.0.241 - 192.168.0.242 | 192.168.0.243 |

# Network Mapping

ACME Inc. requested that a penetration tester be given access to their network directly using a virtual machine. The penetration tester used a Kali Linux machine with the IP Address 192.168.0.200 configured.

As there was no documentation about the network, the network had to be mapped as a blank slate. As the penetration tester had a machine already situated in the network, they were able to use a range of tools to help them map the network. To begin with the attacker used the “if config” command to discover the subnet the attacker was a part of.

Fig 3.1 - The results of the “if config” command on the attacker's virtual machine

The netmask as displayed above is 255.255.255.224. This is representative of a /27 subnet. With this knowledge the penetration tester was able to run a Nmap scan on the subnet that the attacker's machine is a part of. Nmap is a commonly used tool in network mapping and penetration testing. It works by scanning the designated IP addresses and sending crafted packets to retrieve information about the targets. The scan used is “nmap -sV 192.168.0.192/27” this is made up of 3 sections. [ref]

- -sV – A flag set for version detection

- 192.168.0.192 - the network address of the subnet

- /27 - the CIDR notation subnet mask

Fig 3.2 - The results of the nmap scan “nmap -sV 192.168.0.192/27” on the subnet

The nmap scan revealed that the IP address 192.168.0.193 is a router, it also revealed that the router is running VyOS telnet. Telnet is a protocol used for remote login to networked devices and servers, it is widely regarded as insecure due to its lack of encryption, weak authentication, unsecured connections and generally outdated technology [ref]. The penetration tester used telnet to attempt to connect to and gain access to the router.

Fig 3.3 - The telnet connection and successful login

Upon connection to the router the penetration tester was asked for login credentials. After some basic research the default VyOS credentials were found online and tested against the router, these allowed the penetration tester access to the router.

Fig 3.4 - Default VyOS credentials found in the VyOS documentation manual

Using the access to the router the penetration tester was able to begin mapping the network, starting by performing the “show ip route” command. This command shows the routing table for the router, displaying a range of connections over the network.

Figure 3.5 - The routing table as shown on the 192.168.0.193 router

Following the new information discovered nmap scans were run across each of the subnets shown in the routing table. Due to the number of scans this required the scans will be shown when appropriate for discussion. These scans can be found in Appendix B.

Another command used on the VyOS router is the “show config” command. This command displays the current configuration of the device.

Figure 3.6 - The results of the “show config” command

This displays multiple pieces of critical information to the attacker. It first displays 2 other connections on the router, 192.168.0.225 and 172.16.221.16, after further investigation it was determined that these were interfaces on the router. Using the CIDR notation the attacker can determine the purposes of the IP address subnets. The IP address subnet 172.16.221.16 is a class B address with a /24 subnet meaning it can host many addresses, contrarily the IP address subnet 192.168.0.225 is a class A address with a /30 subnet meaning there are only 2 useable host addresses. This indicates that the 192.168.0.225/30 subnet is being used as a bridging subnet to connect to another router. This helps to build a picture of the network as it can be safely assumed that there is at least one other router within the network.

Further information we can find from this statement includes the use of OSPF (Open Shortest Path First) protocol, a protocol designed to find the best path to each IP address in a network [ref]. Finally, the ethernet ports that IP Addresses are associated with are shown which will help create a clearer picture of the network.

Using the information from the “show config” command a nmap scan can be performed on the 172.16.221.0/24 network. This will identify other devices on the network within the subnet.

Figure 3.7 - the results of the nmap scan on the 172.16.221.0/24 subnet

This scan displays one other device present on the network within the subnet, notably both port 80 and 443 are open and running the http protocol, this strongly suggests that the device is a web server. To test this the IP address is entered into the search bar of a web browser.

Figure 3.8 - The web page hosted on 172.16.221.237

Having explored one of the subnets displayed when using the “show config” command on the 192.168.0.193 router, nmap was used to scan the 192.168.0.224/30 subnet.

Figure 3.8 - Nmap scan results on 192.168.0.224/30 subnet

This scan gives a wealth of information to the attacker including showing that both useable host addresses on this subnet are in use and that both are routers running on the same VyOS system. It can be assumed that the router located at the 192.168.0.226 is a separate router from the previously explored one and that subnets with the CIDR notation of /30 are being used as bridging networks. To confirm this theory, telnet is used to connect to the router on 192.168.0.226.

Figure 3.9 - Telnet connection to router on 192.168.0.226 and login with default credentials

As shown above, this router can also be exploited using the default VyOS credentials, suggesting that all the VyOS routers on this network use these credentials and the “show config” command is used to confirm the theory about bridging networks.

Figure 3.10 - results of the “show config” command on the router located at 192.168.0.226

The “show config” command strengthens the theory that /30 subnets are being used as bridging networks between routers. This makes mapping the network much easier as there is now a consistent rule to follow to discover new devices. It also displays other IP addresses connected to the ethernet ports on the router. On the eth1 port the 192.168.0.33/27 network is connected, this network is too large to be used as a bridging network and likely has other devices connected to it. To explore this subnet a nmap scan can be used.

Figure 3.11 - results of the nmap scan “nmap -sV 192.168.0.32/27”

The nmap scan displays another device discovered on the network, notably the open ports on this discovered device show that it has a file share and SSH enabled, exploitation of these will be covered later.

On the eth2 port 192.168.0.229/30 is connected, this subnet is similar to the bridging network discussed prior and is likely connected to another router, to test this a nmap scan can be performed on the subnet using the command “nmap -sV 192.168.0.228/30”.

Fig 3.10 - nmap scan results on the 192.168.0.228/30 subnet

This scan displays the same results for both hosts found indicating that the device found on the 192.168.0.230 IP address will be a VyOS router, telnet can be used to connect to and exploit this by using the default credentials enumerated prior.

Fig 3.11 - telnet connection to router on 192.168.0.230

Figure 3.12 - results of the “show config” command on the router on 192.168.0.230

The results of the “show config” command confirms the bridging network on the 192.168.0.228/30 subnet. The eth1 port on the router is connected to 192.168.0.129/27 subnet. This subnet is large enough to have 30 useable hosts; this suggests it is used for devices instead of bridging. To test this a nmap scan was performed on the subnet using “nmap -sV 192.168.0.128/27”.

Figure 3.13 - The results of a nmap scan on the 192.168.0.128/27 subnet

This scan reveals another device connected to the network. It is also running network file share and SSH. This also confirms that subnets with the CIDR notation /27 are not likely to be bridging subnets.

Finally, a nmap scan is performed on the subnet 192.168.0.232/30 which is connected to the eth2 port, it is assumed that this is a bridging network.

Figure 3.14 - the results of a nmap scan on the 192.168.0.232/30 subnet

This scan reveals critical information about the network as it reveals that despite being a /30 network there does not appear to be a second host on the subnet. Using the established idea that /30 networks are used for bridging there are 2 possible conclusions.

- This is the end of the network and there's nothing more to map or explore

- There is a firewall in place blocking connections from the rest of the network

Earlier we scanned the network based on the IP routing table, this revealed a connection to the subnet 192.168.0.240/30, as this has not been discovered it is safe to assume there is a firewall in place blocking traffic.

In order to explore the rest of the network gaining access beyond the firewall is necessary, a common method of doing this is editing the firewalls settings to allow certain or all packets through the firewall. A way to edit these settings is via the website used to configure them, this is hosted on the IP address 192.168.0.234. Accessing the website can be achieved using port forwarding. Port forwarding is an application of network address translation (NAT) that redirects a communication request from one address and port number combination to another while the packets are traversing a network gateway, such as a router. This technique allows remote computers (for example, computers on the Internet) to connect to a specific computer or service within a private local-area network [ref]. To port forward a shell must be created on a device within the network of the firewall as discussed prior during the nmap scanning of the subnets the IP address 192.168.0.242 was discovered which is within the firewall. The results of the nmap scanning on this subnet can be seen below.

Figure 3.15 - Results of the nmap scan on the 192.168.0.240/30 subnet

As shown the nmap scan discovered that this address was running http which suggests it is a web server, when browsed to it shows a web page confirming the belief that this IP address hosts a web server. Nikto is a web application vulnerability scanner that can be used to find potential exploits on the 192.168.0.242 IP address. This is performed by running the command “nikto -h 192.168.0.242”. This command is made up of the following:

- -h [IP address] is the hostname followed by the address, this shows nikto what to scan.

Figure 3.16 - Nikto scan results for 192.168.0.242

The results of the nikto scan show that the web server address may be vulnerable to a “shellshock” attack. Shellshock will be further explored in the security weaknesses section. This can be accessed using the Metasploit framework built into the Kali operating system.

Figure 3.17.1 - Search for shellshock in the Metasploit framework

Figure 3.17.2 - Using the correct exploit

Figure 3.17.3 - Setting options and running the exploit

As shown above, the only required options to be set are:

- RHOSTS – the target IP address

- TARGETURI – The file path where the exploit should be completed, found in the nikto scan

Running the exploit opens a meterpreter shell on the target machine, allowing us to continue with the exploit by running the “portfwd” command on 192.168.0.242.

Figure 3.18 - Creating the port forward

The commands above create the link by using the following parts:

- -l 500 – adds listening ports to the attacker's kali machine allowing the passage of traffic to that port

- -p 80 – indicates the target port for the remote machine

- -r 192.168.0.234 - indicates the destination IP address of the target, in this case the IP address where the firewall is located.

Once this has been executed the firewall can be browsed to using the ‘localhost:500’ in the browser on the attacker’s machine.

Figure 3.19 - The login page for the firewall after port forwarding has been performed

After some brief research the default pfsense credentials to the firewall are found, upon testing these access is granted.

Figure 3.21 - Default credentials of the pfsense firewall found online

Figure 3.22 - Firewall dashboard after logging in

After signing in the firewall settings can be reconfigured to allow ICMP, TCP and UDP traffic through allowing for access to the full network, we can also view the packet paths.

Figure 3.23 - ICMP packet pathway

The ICMP packet pathway confirms the relation between .234 to .233 further confirms the location of the firewall in the network.

Figure 3.24 - Interfaces of the firewall

Taking a closer look at the firewalls interfaces reveals multiple other IP addresses not located when performing nmap scanning. This suggests another scan of the network should be performed after the firewall has been reconfigured.

After logging in, the attacker has full access to the settings of the firewall rules. These rules enforce how some packets work across the network. For example, if ICMP packets are being blocked no devices outside of the firewall would be able to ping devices within the firewall. To continue mapping the network these settings must be reconfigured with special attention being given to the TCP/UDP packets. This is because nmap relies on TCP packets to scan its target machines. The process of editing the firewalls rules can be found Appendix D. Below the reconfigured rules of the firewall is shown.

Figure 3.25 - The reconfigured firewall rules, allowing ICMP, TCP and UDP packets to be sent and received across the full network.

Now that TCP/UDP traffic is allowed across the entire network, nmap scans should be performed against all the subnets discovered on the IP routing table shown in Figure 3.5.

Most of the subnets saw no change after the reconfiguration of the firewall however, 2 subnets were of note. In the initial scan of the network, subnet 192.168.0.64/27 and 192.168.0.96/27 were shown to have no active hosts despite being mentioned in the routing table.

Figure 3.26 - The initial nmap scan on subnet 192.168.0.64/27

Figure 3.27 - The initial nmap scan on subnet 192.168.0.96/27

It is safe to assume that these subnets were behind the firewall and therefore went unnoticed prior to the changing of the rules on the firewall, upon a second nmap scan after editing the firewalls configuration multiple hosts were found.

Figure 3.28 - The results of the nmap scan on subnet 192.168.0.64/27 after editing the firewall rules.

Figure 3.29 - The results of the nmap scan on subnet 192.168.0.96/27 after editing the firewall rules.

The results of this nmap scan show that another router has been discovered, this expands the network map again. It is assumed that this router is the same as the previously exploited routers as it is also running VyOS and telnet. As such this router can be exploited using the same techniques as previous routers. First using telnet to connect to the router and attempting to use the default credentials to login. Notably, the IP address used to connect to the router is 192.168.0.97, as it was discovered when exploring the firewall that the LAN interface is placed on 192.168.0.98 suggesting that they are connected.

Figure 3.30 - Telnet connection and login to VyOS router on 192.168.0.97

Figure 3.31 - Results of the “show config” command on the router on 192.168.0.97

The configuration of this router confirms the suspicion that both the 192.168.0.64/27 and 192.168.0.96/27 routers are placed behind the firewall and suggests a natural endpoint for the network as shown in the scans in Figures 3.25.1 & 3.25.2 there are no further routers to explore and no further /30 networks which have been used throughout the network for bridging to other routers/areas of the network.

Further exploration of the network using SSH vulnerabilities which will be further explored in the security weaknesses section of the report revealed an unnoticed IP address in 13.13.13.12. This was discovered by performing the “ifconfig” command on a shell on the device 192.168.0.34. “ifconfig” is a command that shows direct connections to a device.

Figure 3.32 - Results of the “ifconfig” command on the device 192.168.0.34

This address initially blocked nmap scans on it, denying crucial information to the attacker.

Figure 3.33 - nmap scan being blocked by 13.13.13.12

The solution implemented was to use SSH Tunnelling to allow the scan, this weakness will be discussed in the security vulnerabilities section. After the tunnel was successfully implemented a nmap scan was performed on 13.13.13.12.

Figure 3.34 - Results of the nmap scan on 13.13.13.12

The successful nmap scan shows that this device is a Linux PC and stands alone, ending the network discovery process.

One notable feature of the network is the use of OSPF (Open Shortest Path First) a link-routing protocol focussed on finding the shortest path to each IP address across the network. This network features 4 OSPF routers of 1.1.1.1, 2.2.2.2, 3.3.3.3 and 4.4.4.4.

# Security Weaknesses

## Exploitation

Using the prior techniques explored in the network mapping phase of the test against Acme Inc. The exploitation phase begins using the previously discussed nmap scans which display the open and active ports and discover other addresses on the network. The information enumerated in the nmap scans is useful for the attackers as it allows the attacker to tailor the exploits they will use to the services running on the devices. The first scan run was on the 192.168.0.192/27 network which displayed a connection on 192.168.0.210 using the NFS (Network File Sharing) service. Network file shares can be exploited if they are not properly configured. To perform this exploit, the following command was used.

Figure 4.1 - The results of the showmount –e 192.268.0.210 command.

This result can be broken down to 2 main parts to understand the output:

- “/” The slash character dictates the highest file directory that can be accessed in the network file share. In this case as it is only the “/” character the root directory is the highest accessible directory

- “192.168.0.*” The IP address range provided denotes the IP ranges that are allowed to access the network file share. An asterisk (*) means that all addresses in the class c IP address range 192.168.0.X can access this file share.

Figure 4.2 - The command used to create the mount1 directory, where the network file system will be mounted.

Figure 4.3 - Mounting the network file system and then accessing and viewing it

The “mount -t nfs 192.168.0.210:/ ./mount1” is used to mount the network file system so that it can be browsed through like a normal directory.

As the network file system allows for any address on the 192.168.0.X range access to the root directory (shown by the “/” character) the attacker can gain access to sensitive files via the Kali machine on the network, experienced attackers are able to navigate the root directory efficiently and cause major damage to the network. A major target in attacks like these is the “/etc” directory. This directory is the location where passwords and usernames are stored in a Linux system. Linux uses 2 files to store passwords, the “shadow” file and “passwd” file. This system was developed to combat password cracking as when a system has shadow passwords enabled, the password field in /etc/passwd is replaced by an “x” and the user's real encrypted password is stored in /etc/shadow. Because /etc/shadow is only readable by the root user, malicious users cannot crack their fellow users' passwords (Bassil. Y, 2012). A way to exploit this is using the unshadow command in Linux as shown below.

Figure 4.4 - The unshadow command being used on the /etc/passwd and /etc/shadow files 192.168.0.210, then outputting the results to a text file.

Figure 4.5 - The unshadowed passwords stored in a new text file

The unshadow command, shown in Figure 4.2.1, merges the /etc/passwd file with the /etc/shadow file generating a text file that can be used along with the password cracking tool “John the ripper” to crack the passwords on the device.

Figure 4.6 - John the ripper tool used to crack the passwords on the account

As shown in Figure 4.6 John the ripper was able to crack the administrator password, allowing the attacker administrative privileges across the network. To test this theory the attacker used SSH to access the device on 192.168.0.210.

Figure 4.7 - gained access to the 192.168.0.210 machine using SSH and admin credentials.

As shown above, the credentials found using John the ripper allow administrator access to the machine on 192.168.0.210. The attacker tested these credentials against SSH on every other device with it enabled on the network and was able to gain access to the following machines:

- 192.168.0.34

- 192.168.0.130, but only after connecting to 192.168.0.34

- 13.13.13.12, but only after connecting to 192.168.0.34

Another issue with the SSH protocol implementation is the lack of root user authentication. By using the previously enumerated xadmin account, the root password can be changed.

Figure 4.8 - The root password being changed using the xadmin account.

The root account password was set to toor for simplicity. Exploiting this vulnerability allowed the tester to attempt to gain access to the machine located on 13.13.13.12 through the process of SSH tunnelling. It was initially confirmed that the device on 13.13.13.12 could not be pinged by the testers provided Kali machine.

Figure 4.9 - Attempt to ping machine located at 13.13.13.12 from the testers machine failed

To confirm that SSH tunnelling was possible the tester used SSH to connect to the 192.168.0.34 machine which is directly connected to the 13.13.13.12 machine and attempted to ping it.

Figure 4.10 - The successful ping on the 13.13.13.12 machine from 192.168.0.34

This confirmed that SSH tunnelling would be possible. The tester then used the SSH access to 192.168.0.34 to access the SSH configuration settings.

Figure 4.11 - Browsing to and accessing the SSH configuration settings

The tester then edited the configuration file to allow the root user to log in with the edited password instead of requiring a public key, the tester also added a line in the configuration called “PermitTunnel” which allows the tester to create an SSH tunnel.

Figure 4.12 - The edited authentication section of the SSH configuration file

The tester was able to use the root account to log into the 192.168.0.34 using the new credentials and created a tunnel by using the “ssh -w0:0 root@192.168.0.34” command.

Figure 4.13 - Root login to the SSH service on 192.168.0.34 creating a tunnel

To confirm the tunnel had been created the tester used the “ip addr” command on both the kali machine and the 192.168.0.34 machine using SSH. The tester then added an IP address to the tunnel from the 192.168.0.34 machine and set the link to be active.

Figure 4.14 - commands are used to provide an IP address to the tunnel on 192.168.0.34 and to set the link to be active

The same commands are used to provide an IP address to the testers machine’s interface of the tunnel and set the link to be active on the tester machine interface of the tunnel.

Figure 4.15 - Commands are used to set the IP address of the tunnel on the kali machine and set the link to be active

The tunnel was successfully set up however forwarding traffic cannot occur as IPV4 routing is not active by default. To edit this the tester used the 192.168.0.34 machine and the echo command to edit the file controlling IPV4 routing to enable it.

Figure 4.16 - IPV4 routing file is edited to allow forwarding of traffic

Finally, the tester used the route add command so that the traffic will be routed through the tunnel to the tester’s machine.

Figure 4.17 - the tunnel traffic is routed correctly

The tester was able to perform a nmap scan on the machine on 13.13.13.12 and was able to discover SSH running on it.

Figure 4.18 - the nmap scan results on 13.13.13.12

The tester used the previously enumerated SSH credentials to log into the SSH service running on 13.13.13.12 which allowed for access to sensitive files such as the password and shadow files.

Using the information enumerated by the nmap scan against each subnet, multiple other devices were found to have Network File System active on them, mounting and accessing was attempted on all of them.

Figure 4.19 - The network file system that is accessible on 192.168.0.34

Figure 4.20 - The network file system that is accessible on 192.168.0.130

As shown in figures 4.19 and 4.20 the devices on 192.168.0.34 and 192.168.0.130 had accessible network file systems however, no information could be gained from them as the highest accessible directory was the /home/xadmin folder.

Figure 4.21 - The network file system that is accessible on 192.168.0.66

The network file system found on 192.168.0.66 is accessible and allows root directory access allowing the attacker to attempt to crack passwords similar to the exploit used on 192.168.0.210. However, while attempting to crack the passwords no results were found using the standard wordlist supplied by john the ripper indicating that the passwords may be very strong and hard to crack or that more time was required to break into them.

The device on 13.13.13.12 refused to connect.

Another device discovered by nmap was a web server on the IP address 172.16.221.237. A commonly used web application vulnerability scanner is Nikto which can be used to scan the web server.

Figure 4.22 - The results of a Nikto scan on the web server located at 172.16.221.237

A notable result from this scan is the ability to brute force file names. This potentially allows the attacker to find hidden files or web pages which they should not be able to access. A notable tool for enumerating these files or web pages is dirb, which works by using wordlists of commonly used files and web page names against the target. The full dirb scan can be found in Appendix A.

Figure 4.23 - Cropped dirb scan showing login page found

Within the output of the dirb scan, a wordpress login page was found. Wordpress is a notoriously vulnerable website technology that is commonly exploited and attacked. To begin with, the username of an administrator was enumerated due to wordpress’s poor error handling.

Figure 4.24 - Username enumerated due to improper error handling

With the knowledge of an administrator password, attempts at brute forcing the password can begin. A tool used for this is WPScan, a web application testing tool specifically designed for wordpress based websites, it also has a built-in brute force password cracker.

Figure 4.25 - Password for “admin” account found using wpscan and rockyou.txt

Using WPScan’s password brute forcing tool and the wordlist rockyou.txt (A large file containing many common passwords) the credentials for the admin account were found.

Another web server discovered on the network is hosted on the address 192.168.0.242. As the attacker used it prior to reconfigure the firewall it is known to be weak to the shellshock vulnerability. Another use of the meterpreter shell is that it can be upgraded to a command shell on the target web server allowing the attacker to dump passwords.

Figure 4.26 - A command shell is opened on the target 192.168.0.242 and the /etc directory is browsed to

As shown in figure 4.26 the attacker was able to get access to the /etc directory where the passwd and shadow files are held. To unshadow and dump these files a hashdump exploit in Metasploit called “/post/linux/gather/hashdump” was used.

Figure 4.27 - The shell is backgrounded and the “post/linux/gather/hashdump” exploit is loaded

Figure 4.28 - The previously backgrounded session is used, and the command is run creating an unshadowed file on the attacker's machine.

The exploit creates an unshadowed text file on the attacker's machine; this allows the John the ripper tool to attempt to crack the passwords.

Figure 4.29 - Passwords found for root and xweb users

The passwords for the web server on 192.168.0.242 for xweb and root were successfully cracked, this could lead to the attacker having high level access to the web server and being able to view, deface or use the website to harvest credentials from legitimate users. Administrators can also upload files which may include malicious files such as reverse shell scripts and malware among others.

Another IP address located using the nmap scan on the 192.168.0.192/27 subnet was 192.168.0.193. The nmap scan indicated that this device was a router and was also running the service telnet. The tester used telnet to connect to the router and then attempted to gain access using the default credentials which was successful.

Figure 4.30 - Telnet connection and successful login using default credentials

This presents another major security issue in using default credentials on any router may allow a malicious actor to seriously damage the network. The tester attempted to further exploit this by attempting to gain access to the root account which is a superuser account with virtually unlimited access to all programs, files, and resources on a system (IBM, 2025).

Figure 4.31 - Superuser access being granted without a password

This vulnerability ties into further problems with default credentials being used across the network including on the firewall and on all other VyOS routers that were discovered.

As discussed previously the machine found behind the firewall 192.168.0.66 was susceptible to NFS vulnerabilities like unshadowing the passwords stored on the system. When attempting to crack the passwords stored on this machine the tester found that they were at least relatively strong and would require a long time and a strong wordlist to crack. Due to time constraints this would not be possible so instead a different vulnerability was attempted.

A common work around to not knowing SSH credentials is to abuse the RSA certificates of the device. The method the tester used to attempt this was to generate a public and private key pair. Public key cryptography is a method of encrypting or signing data with two different keys and making one of the keys, the public key, available for anyone to use. The other key is known as the private key. Data encrypted with the public key can only be decrypted with the private key (Cloudflare, N.D.).

Figure 4.32 - The generation of the public-private key pair by the attacker

The attack involves copying the attackers public key to the target machine allowing the attacker to log in using the private key, eliminating the use for a password. The tester attempted to use the NFS service on 192.168.0.66 to copy the public key to the ssh configuration file. This step of the exploit was successful however, when the tester attempted to connect to SSH the connection was refused.

### Mitigation

A common issue across the network is the user of default credentials, this appeared both in the routers and the firewall and allowed the tester to bypass proper authentication. To configure a password on the VyOS routers the following commands should be used:

- Connect to the router and log in.

- Type “configure terminal” to enter the router’s configuration mode.

- Type “line vty 0 15” to switch from global configuration to line-by-line configuration mode, this command targets lines 0-15 which covers telnet.

- Type “password [password]” to set a password for connecting to the selected lines.

- Type “login” to require the password to be entered before access is granted.

- Save changes by exiting configuration mode and entering the line “set running-config startup-config".

A password should also be set on the routers to authenticate superuser account access, this will restrict access to the most privileged level of account and adds an additional layer of security to the network. This can be performed using the line:

- “enable secret [password]” which sets a password to access higher privilege mode.

- Save changes by exiting configuration mode and entering the line “set running-config startup-config".

A password should also be set on the pfsense firewall ensure that only network administrators and other authorised users can access the firewall as making changes to the configuration of the firewall makes the network significantly more vulnerable. According to Netagate this can be performed on the web interface as follows (Netgate, N.D.):

- Navigate to the web interface of the firewall, in this network it was found on 192.168.0.34.

- Log in to the firewall

- Navigate to the System > User manager section

- Click the pencil icon next to the admin user

- Enter and confirm the new password

- Save and exit

Another issue discovered on the network is the exploitable NFS system. Whilst not all machines had NFS enabled or were exploitable both 192.168.0.210 and 192.168.0.66 allowed the tester access to root directory files allowing for passwords to be stolen and attempted to be cracked. To prevent exploits occurring access to the NFS should be restricted to only those who require it. Ensure that the option no_root_squash is not set on any files which are exportable. Configure the system so that the least amount of privileges necessary are exported such as not allowing users to write to files if it is not required (IBM, 2025). If possible, a more secure method of file sharing should be implemented instead of NFS such as using Secure File Transport Protocol which uses encryption and SSH to transfer files.

In general, a stronger password policy should be implemented such as encouraging users of the network to create longer passwords with symbols, numbers and a mix of upper case and lower-case letters. Another solution is to use different passwords for each device on the network meaning that exposed credentials on one device does not allow for exploitation of all other devices on the network.

Shellshock is a vulnerability in Bash that allows remote code execution without confirmation. A series of random characters, () { :; }; , confuses Bash because it doesn't know what to do with them, so by default, it executes the code after it. (Das N., 2020). This vulnerability is present in all versions of bash from Version 1.14 until and including Version 4.3. To ensure that this vulnerability is not present within the system the version of bash should be updated anywhere it is present. This can be performed using the command:

- sudo apt install --only-upgrade bash

To ensure that bash has been correctly updated, a nikto scan should be performed confirming that the shellshock vulnerability is not present.

Finally, out of date and insecure services such as telnet should be removed from the network and replaced with more secure services such as SSH.

# Network Design Critical Evaluation

The Acme Inc. Network has multiple positive attributes in both the security and networking aspects however, this is soured by the glaring lack of security, multiple critical vulnerabilities and lack of robust networking.

In general, the network makes efficient use of subnets with strong points being the use of /30 networks as bridging networks. As /30 networks can only have 2 useable hosts it is both efficient and logical to use them to bridge between points on the network. Another positive use of subnetting is seen in the /27 subnets which allow for many more useable hosts to connect. It is important to point out the some of the /27 subnets only make use of 2 useable hosts however it is important to allow for more connections for expansion of the network without having to re-subnet the network.

The network also attempts to make use of OSPF protocol which allows for improved efficiency across the network.

The network had some good security ideas such as the use of a firewall which was able to hide a section of the network by blocking ICMP and TCP pings which hides sections of the network from both ping scans and Nmap's TCP scans. To improve upon this a firewall should be used to cover more of the network meaning there is less visible targets for attackers when initially surveying the network.

Another attempt at security was made by having some of the devices that were running NFS preventing unauthorised access. Further implementation of this would greatly improve security of the network as cracking passwords and file traversal would not be so easily completed. Deviation in passwords across different devices on the network would allow for a generally safer network as one device being compromised wouldn't expose the entire network.

There are some issues with the networking concepts used throughout the network such as the 192.168.0.96/27 subnet being used as a bridging network despite its large number of possible connections. Another issue arises with the device on 13.13.13.12 as this utilises a class A IP address with a /24 subnet that allows for 254 possible connections of which only one is used wasting a large number of possible hosts and considering where the machine is located it is unlikely that expansion of that area of the network is planned. Another issue is the use of the 172.16.221.0/24 subnet which uses a class B IP address and allows for 254 possible hosts of which 2 are used. If these hosts are being used for future expansion, then the use of this subnet is understandable but currently a different subnet should be used such as a /27 subnet.

A major security issue is the use of default credentials across the network. This allowed the tester to access routers and the firewall effectively making the security provided by the firewall useless. The credentials on all routers and the firewall should be changed immediately. Furthermore, the credentials set up on the PC’s and web servers are incredibly weak and easily compromised. These should be changed in accordance with Acme Inc. Implementing a strong password policy. The password policy should at minimum satisfy the following requirements:

- Passwords must make use of a mixture of upper case and lower-case letters.

- Passwords must make use of at least one number and one symbol.

- Passwords should have a minimum length of 8 characters.

- Passwords should eventually expire and require changing.

Further security issues include the web servers being exploitable by the vulnerability “shellshock” which allows attackers full access to the web servers and can allow for full access to the network.

Another security measure that can be taken to protect against attacks is the inclusion of an Intrusion Detection System (IDS). An IDS is a network security tool that monitors network traffic and devices for known malicious activity, suspicious activity or security policy violations (IBM, N.D.). This could allow Acme Inc. to monitor and log potential security risks and alert them to attacks that may occur.

Finally, the structure of the network may cause issues. Acme Inc. Uses a “Bus Topology” network structure. A bus topology network is a type of network topology in which all the devices are connected to a single cable which is called the backbone of the network. This structure is effective for a small network as it is Cost effective, simple to install and requires little cabling (GeeksForGeeks, 2025). A major issue with this structure is that it relies on the routers on the “backbone” to maintain activity or the whole network becomes unusable. This makes the network especially vulnerable to Denial-of-Service attacks. It also makes the network generally less robust. To circumvent this problem Acme Inc. Should consider re-structuring their network to utilise a “mesh topology”. A mesh topology is where every device is connected to another device via a particular channel. Every device is connected to another via dedicated channels. These channels are known as links (GeeksForGeeks, 2025). The advantage of this topology is that it allows for more robustness as one device going down will not affect the rest of the network.

# Conclusions

The Acme Inc. Network contained mostly negative networking and security aspects which could lead to significant problems in the future. Aspects such as subnetting were generally well implemented however, the network lacks security features such as the use of default or weak passwords, vulnerability to known attacks like shellshock and lack of protection against NFS access. It may be assumed that the previous network administrator was not well versed in cyber security. It is recommended that the documentation provided by the tester should be kept and a new network administrator should be hired to improve the security of the network and the implement the solutions provided.

# References

Bassil Y. (2012) Windows And Linux Operating Systems From A Security Perspective. Available at: https://arxiv.org/abs/1204.0197

Cloudflare (no date) How does public key encryption work. Available at: https://www.cloudflare.com/en-gb/learning/ssl/how-does-public-key-encryption-work/ (Accessed: 11 December 2025)

Das N., (2020) The ShellShock Vulnerability Available at: https://www.exploit-db.com/docs/48112

GeeksForGeeks (2025) Difference between Star topology and Bus topology. Available at: https://www.geeksforgeeks.org/computer-networks/difference-between-star-topology-and-bus-topology/

IBM (2025) Root account. Available at: https://www.ibm.com/docs/en/aix/7.3.0?topic=passwords-root-account

IBM (no date) What is an intrusion detection system (IDS)? Available at: https://www.ibm.com/think/topics/intrusion-detection-system (Accessed: 13 December 2025)

Netagate (no date) Manage local users. Available at: https://docs.netgate.com/pfsense/en/latest/usermanager/users.html#creating-and-editing-users (Accessed: 11 December 2025)

Network Academy (no date) What is OSPF? Available at: https://www.networkacademy.io/ccna/ospf/what-is-ospf (Accessed: 21 October 2025)

Nmap (no date) Service and Version Detection. Available at: https://nmap.org/book/man-version-detection.html (Accessed: 13 October 2025)

N. Verma., M. Kashyap., A. Jha., (2018) ‘Extending port forwarding concept to IOT’, 2018 International Conference on Advances in Computing, Communication Control and Networking (ICACCCN), pp. 37-42, Available at:10.1109/ICACCCN.2018.8748430

UK Cyber Security (no date) What makes telnet vulnerable? Available at: https://www.ukcybersecurity.co.uk/blog/news-advice/what-makes-telnet-vulnerable/ (Accessed: 17 October 2025)

VyOS (no date) Installation. Available at: https://docs.vyos.io/en/latest/installation/install.html (Accessed: 17 October 2025)

# Appendices

## Appendix A – Dirb scan

## Appendix B – Nmap scan

Nmap scans on the subnets that returned no results after the edited firewall configuration

## Appendix C – Subnet Calculations

Subnet mask: 255.255.255.0 or /24

In binary: 00000000

Flip the bits to get wildcard mask:

11111111

Convert to decimal and subtract broadcast address

255-1 = 254

254 useable hosts

Subnet mask: 255.255.255.224 or /27

In binary: 11100000

Flip the bits to get the wildcard mask:

00011111

Convert to decimal and subtract the broadcast address:

31-1 = 30

30 useable hosts

Subnet mask: 255.255.255.252 or /30

In binary: 11111100

Flip the bits to get the wildcard mask:

00000011

Convert to decimal and subtract the broadcast address:

3 - 1 = 2

2 useable hosts

## Appendix D – Firewall configuration